Technical Specification: SIF/YXT Protocol – One Wire

Document Version: 1.0

Author: VECS System Engineering

Target Hardware: Interception & Control Module

Status: Decoded & Validated

System Overview

The SIF (Serial Interface Format) or YXT Protocol is a single-wire, unidirectional asynchronous pulse-width ratio encoded communication interface. It is primarily utilized by Chinese OEM e-bike motor controllers to continuously stream telemetry data (speed, battery state, fault flags, and mode indicators) to an attached liquid crystal display (LCD) unit.

Physical Layer (PHY)

- Physical Wire: Single conductor (standard color code: Purple), common reference ground (Black).

- Signal Voltage: 3.3V TTL Logic (Idle state: HIGH / 3.3V).

- Operating Impedance: Driven in push-pull configuration by the motor controller, or via active weak pull-up.

- Effective Average DC Line Voltage: ≈1.5V-1.8V during active transmission (due to ≈50% duty cycle).

Clocking & Bit Encoding Architecture

Communication timing is determined by a base oscillator timebase (Tosc). Data bits are defined by the duration of the active-LOW voltage state relative to Tosc.

Base Parameter

Tosc=25 μs

Modulation Scheme (Pulse-Width Ratio)

Every bit transmission occupies a fixed duration frame of 96 Tosc (2.4 ms), providing an effective throughput of ≈416.6 bps.

Plaintext

Active-LOW Period          Passive-HIGH Period         Total Bit Period

Logic '1':   32 * Tosc ( 800 us)   +    64 * Tosc (1600 us)     =   96 * Tosc (2.4 ms)

Logic '0':   64 * Tosc (1600 us)   +    32 * Tosc (800 us)      =   96 * Tosc (2.4 ms)

Plaintext

Timing Waveform (Bit 1 vs Bit 0):

Bit '1' (1:2 Ratio):

HIGH --+                   +------------------------+

|                   |                        |

LOW   +-------------------+                        +---

<-- 800us (LOW) ----><---- 1600us (HIGH) ---->

Bit '0' (2:1 Ratio):

HIGH --+                                            +---

|                                            |

LOW   +--------------------------------------------+

<------- 1600us (LOW) -------><-- 800us (HIGH) ->

Frame & Packet Specification

A complete packet transmission consists of 97 physical bits: a single Sync Preamble followed by a 12-byte payload transmitted Most Significant Bit (MSB) first.

Plaintext

+-----------------------+--------------------+--------------------------------+--------------------+

| Sync Preamble         | Header Bytes (0-1) | Telemetry & Flag Bytes (2-10)  | Checksum Byte (11) |

| 25ms LOW / 1ms HIGH   | DATA0 - DATA1      | DATA2 - DATA10                 | DATA11 (XOR)       |

+-----------------------+--------------------+--------------------------------+--------------------+

Frame Timing Elements

| Segment | Formula Multiplier | Duration (μs) | Voltage Line State | Purpose |
| --- | --- | --- | --- | --- |
| Sync LOW | ≥992 Tosc | 25,000 μs (25 ms) | LOW (0V) | Receiver Bit-Counter Reset |
| Sync HIGH | ≥32 Tosc | 1,000 μs (1 ms) | HIGH (3.3V) | Preamble Delimiter |
| Data Payload | 12 Bytes×8 bits | 230,400 μs (230.4 ms) | Pulse-Width Modulation | 96-Bit Telemetry Data Stream |
| Inter-frame Gap | — | 30 ms-50 ms | HIGH (3.3V) | Bus Idle Time |

12-Byte Payload Mapping Table

| Byte | Parameter Name | Hex Mask / Values | Bit Field Mapping & Description |
| --- | --- | --- | --- |
| DATA0 | Device ID | 0x08 | Display Hardware Header Identifier (Validated). |
| DATA1 | Protocol ID | 0x61 | Protocol Subtype Identifier. |
| DATA2 | Assist / Mode Register | 0x00 - 0x05 | Pedal Assist Level (PAS 0 to 5) / Riding Mode. |
| DATA3 | Fault Status Register | Bit 5 (0x20) | Bit 5: Throttle Error Icon (1 = Active Fault). |
| DATA4 | Display Feature Control | Bit 0 (0x01) Bit 3 (0x08) Bit 4 (0x10) | Bit 0: Headlight / Backlight Icon Bit 3: Cruise Control Indicator Bit 4: Walk Mode / Push Assist Icon |
| DATA5 | Motion Flags | Bit 2 (0x04) | Bit 2: Reverse Mode Indicator. |
| DATA6 | Reserved / Temp | 0x00 | Controller Temperature / Internal Flag. |
| DATA7 | Reserved / Motor Error | 0x00 | Hall Sensor / Motor Phase Error Register. |
| DATA8 | Speed High Byte (MSB) | 0x00 - 0xFF | Upper 8 bits of raw speed calculation. |
| DATA9 | Speed Low Byte (LSB) | 0x00 - 0xFF | Lower 8 bits of raw speed calculation. Speed (km/h)=DATA8≪8∣DATA910 |
| DATA10 | Battery Level | 0x00 - 0x64 | Battery Charge Percentage (0% to 100%). |
| DATA11 | Frame Checksum | 0x00 - 0xFF | Dynamic 8-bit XOR Checksum. |

Checksum Calculation Algorithm

The checksum byte (DATA11) guarantees frame integrity. The receiver calculates an 8-bit Exclusive OR (XOR) over the preceding 11 data bytes (DATA0 through DATA10). If the calculated value does not match DATA11, the display rejects the entire packet.

DATA11=i=010DATAi=DATA0⊕DATA1⊕DATA2⊕…⊕DATA10

Reference C Implementation

C

uint8_t calculateChecksum(const uint8_t *frameBuffer) {

uint8_t checksum = 0;

for (uint8_t i = 0; i < 11; i++) {

checksum ^= frameBuffer[i];

}

return checksum;

}